# CoCIS Smart Control and Monitoring System

Project summary and guide to the source files  |  ESP32  |  ESP-IDF (FreeRTOS, C)  |  Wokwi simulation

# 1. What the system does

The system automates security and lighting for the pilot area described in the coursework: the main entrance of the CoCIS building and two lecture rooms. It runs on an ESP32 microcontroller programmed with ESP-IDF and is simulated in Wokwi.

- Access control at the main entrance. People tap an RFID card at a separate entry reader or exit reader. Each scan is checked against the registered users, the access schedule and the list of people already inside. If access is allowed, the matching door (a servo motor) opens for 3 seconds. An LCD screen, LEDs and a buzzer show the result.

- Occupancy monitoring. The system always knows who is inside the building, when each person entered, and how many people are present.

- Automatic lighting. Each lecture room has a motion sensor (PIR) and a light sensor (LDR). Lights switch on only when the room is occupied and too dark, and switch off after a period with no motion.

- Data storage. Every access event, room event, administrator action and system event is saved to an SD card, with a daily report.

- Management interface. A serial console provides menus and commands for status, occupancy, room states and logs. Changing the schedule or settings requires an administrator login.

- Synchronization. Three terminal tasks (Entrance, Room 1, Room 2) never change shared data directly. They send messages to a single coordinator task, which owns all building state. Mutexes protect the remaining shared resources, preventing race conditions and inconsistent records.

# 2. Project files

## 2.1 Project root folder

| File | Purpose |
| --- | --- |
| IMPLEMENTATION_PLAN.md | The original plan: requirements checklist, architecture, pin map, team roles and timeline. |
| PRESENTATION_GUIDE.md | Compliance checklist against the coursework PDF, presentation plan, what each member explains, and likely lecturer questions with answers. |
| .github/workflows/ build.yml | GitHub Actions script that builds the firmware in the cloud on every push and produces the downloadable firmware file. |
| .gitignore | Stops build output and the coursework PDF from being uploaded to GitHub. |

## 2.2 Project setup (cocis_smart folder)

| File | Purpose |
| --- | --- |
| diagram.json | The Wokwi circuit: the ESP32 and every connected part and wire. |
| wokwi.toml | Tells the Wokwi simulator in VS Code which firmware files to run. |
| CMakeLists.txt main/CMakeLists.txt | ESP-IDF build instructions: project name, source files and required libraries. |
| sdkconfig.defaults | ESP-IDF settings such as the system tick rate, flash partition size and watchdog. |
| README.md | How to build and run the project, the preset demo cards, and a demonstration script. |
| build/ | The compiled firmware (.bin and .elf) that Wokwi runs. Generated, so not stored on GitHub. |

## 2.3 Source code (cocis_smart/main folder)

| File | Purpose |
| --- | --- |
| app.h | Shared definitions used by every module: pin numbers, data types, message formats, the list of functions, and the lock order that prevents deadlock. |
| main.c | System start-up: creates all queues, mutexes and tasks, and loads the time from the real-time clock. |
| arduino_entry.cpp | Alternative start-up entry used only by the wokwi.com online editor, which builds on the Arduino-ESP32 core and requires setup() and loop(). |
| coordinator.c | The central decision-maker. Grants or denies access, enforces anti-passback (no double entry, no exit without entry), tracks who is inside, keeps statistics and handles emergency mode. |
| entrance.c | Main-entrance terminal. Reads cards (simulated by buttons or the tap command), asks the coordinator for a decision, then opens the door and gives feedback. |
| room.c | Lecture-room terminal (one copy runs for each room). Detects occupancy, controls the lights, uses daylight to save energy, raises intrusion alerts and counts energy saved. |
| hw.c | Hardware drivers: door servos, buzzer, status LEDs, light sensors, room lights and the emergency button interrupt. |
| i2c_dev.c | Drivers for the DS1307 real-time clock and the 16x2 LCD screen, plus the task that keeps the display updated. |
| storage.c | SD card handling: mounting, the logger task (the only task that writes files), a RAM backup if the card fails, the user database, and restoring the occupancy list after a restart. |
| schedule.c | Access hours for each role (student, staff, admin, security), closing-time warnings, the list of people still inside at closing, daily reports and terminal health checks. |
| auth.c | Administrator login: salted and hashed passwords, lockout after three wrong attempts, and automatic session timeout. |
| config.c | Saves and loads system settings (schedule, timeouts, thresholds) in the ESP32's flash memory. |
| cli.c | The serial management console with 23 commands, for example menu, status, who, rooms, logs and schedule set. |
| stress.c | The race and stress tests, which demonstrate that the synchronization prevents lost updates and inconsistent records. |